# Halda M10 Calibration Instructions

Please note: You must have an accurately measured mile before you attempt to re-calibrate your taximeter.

1) Drive to the start the mile

2) Undo two front screws on meter

3) Slide meter off the mounting plate

4) Turn meter over so that you see a bank of red switches no. 1-10

1

Extras

Fare

These screws to be removed

Calibration Switches

Now with a pen or another sharp object, push down on each switch at the end identified as open, until the switches are all level with the white end up at the numbered side.

5) Now slide the meter back onto the mounting plate, you will notice 000 in the fare window, if you have done it properly

6) Drive over your mile and stop when you reach the end

7) On your meter there will be a reading eg.4010

8)Take this figure and divide it into 1638411.

1638400 divided by 4010  = 408.578, ignore all decimals and transfer your figure of 408 into the taximeter by adding up the appropriate switch values to obtain 408 and then push those switches down at the numbered end.

As an example switches 4,5,8,9 add up to 408.

## Switch Values

| ### Switch | ### Value |
| --- | --- |
| 1 | 1 |
| 2 | 2 |
| 3 | 4 |
| 4 | 8 |
| 5 | 16 |
| 6 | 32 |
| 7 | 64 |
| 8 | 128 |
| 9 | 256 |
| 10 | 512 |

Now you can re-fit your meter to the mounting plate.